6. ENGINEERING SYSTEMS REQUIREMENTS

C30 INTERIOR FINISHES

********************************************************************************************************** 
SYSTEMS REQUIREMENTS
INTERIOR FINISHES TEMPLATE 12/08
 
Instructions for using this template: There are template files for each UNIFORMAT Level 2 Group Elements. This template is for Group Element C30-INTERIOR FINISHES. Text such as this is hidden text that will not print when the hidden text box in "Print/Options" is un-checked.
 
The Interior Design Team Member should edit this template for the requirements of the project. The SYSTEMS REQUIREMENTS are intended to define items that are required throughout the facility or on a system wide basis that are common to several rooms. Room-specific requirements are defined in the ROOM REQUIREMENTS section. Coordinate with the lead programmer for ROOM REQUIREMENTS. Editing is required where brackets [ ] appear. Delete all building elements that are not required for the project.    If additional elements or sub-elements are required for the project that do not appear in the template, refer to the NIST UNIFORMAT II publication for additional building element numbers and descriptions. The Uniformat II Work Breakdown Structure can be found at www.wbdg.org/ndbm/ . Coordinate with the PERFORMANCE TECHNICAL SPECIFICATION SECTION C30 to ensure that performance requirements are provided for all of the Building Elements listed here and that paragraph numbering matches.
**********************************************************************************************************

SYSTEM DESCRIPTION
********************************************************************************************************** 
NOTE:    Edit the following paragraph based on the project specific information received from the User.    Fill in the blank with the function of the building, such as residence, barracks, administrative office building, training facility, operations building, fire station, etc..
**********************************************************************************************************

Interior finishes include wall finishes, floor finishes, wall base finishes, and ceiling finishes.

Provide aesthetically pleasing, functional, durable finishes appropriate to the buildings function.      Acoustic properties of materials, as well as durability and ease of maintenance, shall be considered during material selection.    Maximize the use of sustainable materials.

GENERAL SYSTEMS REQUIREMENTS
See "Room Requirements" for specific requirements on "Interior Finishes."

C30 SSPC QP 1 CERTIFICATION

********************************************************************************************************** 
NOTE:    Require the contractor to hold a certificate concerning SSPC QP 1 only when industrial coatings are required on large structural members for facilities such as hangars or other large open buildings with exposed structural steel. Indicate below the surfaces requiring industrial coatings. Painting qualifications for projects such as BEQ's, training facilities, and general administration buildings do not require QP 1 certification. SSPC QP 1 certified Applicators are available in the continental US, Hawaii, Alaska, and Puerto Rico but confirm availability to your job area.
 
For projects that involve removal of paint containing hazardous materials, add requirement for SSPC QP-2 certification in ESR Section F20.
**********************************************************************************************************

[The Project requires industrial coatings on [____] surfaces.    All contractors and subcontractors that perform surface preparation or coating application shall be certified by the Society for Protective Coatings (formerly Steel Structures Painting Council - SSPC) to the requirements of SSPC QP 1 prior to contract award, and shall remain certified while accomplishing any surface preparation or coating application.]

C3010 WALL FINISHES

********************************************************************************************************** 
NOTE:    Consider one of the following sentences, or create your own, to describe the wall finishes of the facility.    The first sentence is applicable to a barracks, or industrial facility, where extreme durability is required.    The second sentence applies to an administrative office building.    The "Room Requirements" reference sentence applies to a facility where the "Room Requirements" or a "Room Finish Schedule" is very detailed as to the wall finish requirements.
**********************************************************************************************************

[All interior wall finish materials shall be concrete masonry.]

[All interior wall finish materials shall be gypsum board. [Provide vinyl [acoustic] [tackable] wallcovering in all conference rooms and entrance lobby], [and] [provide ceramic tile wainscot in all toilet rooms to a height of ___ feet.]

[Mirror wall (one uninterrupted long wall), in fitness area, shall be 28 inches (712 mm) off floor to a minimum height of 8 feet (2438 mm) A.F.F.]

[Solid surfacing wall finishes shall extend from top of shower pan to 84" and shall surround the shower enclosure. Wall finish shall extend from top of tub to 72" and shall surround tub shower. Wall finish shall extend from top of kitchen countertop to underside of wall cabinet.]

[All interior wall finishes shall be as indicated in the “Room Requirements” [Room Finish Schedule] portion of this RFP[ or]

[Conceptual Wall Finish Schedule

SPACE                MINIMUM FINISH REQUIREMENT
Private Office
Open Office
Training Room
Conference Room
Reception
Kitchen/Break Room
Restrooms
Lobby/Entrance
Executive Suite
Corridors
Physical Fitness Area]

C3020 FLOOR FINISHES

********************************************************************************************************** 
NOTE:    Consider one of the following sentences, or create your own, to describe the interior floor finishes of the facility.    The first sentence applies to industrial facilities, where durability and low cost is required.    The second sentence applies to most other facilities.    Use the "Room Requirements" reference sentence if the "Room Requirements" or "Room Finish Schedule" are very detailed as to the floor finish requirements.
**********************************************************************************************************

[Primary floor finish shall be vinyl composition tile, except ceramic tile shall be provided in toilets [and shower areas].

[Primary floor finish shall be [linoleum] [linoleum tile], except ceramic tile shall be provided in toilets [and shower areas].]

[Primary floor finish shall be carpet [tile].

a.    Carpet Requirements

Surface Color:                  Multi-colored and Patterned
Surface Texture:        Loop, Cut/Loop, Tip-shear, Textured Loop
Yarn Weight:                [24][26][28][32] oz. Minimum.
Dye Method:                [Solution Dyed][Yarn Dyed][Piece Dyed]
Backing:                [Industry Standard][Attached Cushion]

[Building entrances, kitchen areas and restrooms shall be porcelain [or ceramic] tile.

Exposed concrete floors shall be coated with a sealer appropriate to the function of the space.      [Colored concrete floor shall be [colored pigment integral to the concrete mix] [applied as a topical dye] [a concrete topping with integral color pigment] [or] [a dry shake pigment application].]

[Floor finishes shall be as indicated in the “Room Requirements” [Room Finish Schedule] portion of this RFP[ or].

[Conceptual Floor Finish Schedule

SPACE                MINIMUM FINISH REQUIREMENT
Private Office
Open Office
Training Room
Conference Room
Reception
Kitchen/Break Room
Restrooms
Lobby/Entrance
Executive Suite
Corridors
Physical Fitness Area]

C3030 CEILING FINISHES

********************************************************************************************************** 
NOTE:    Consider one of the following sentences, or create your own, to describe the interior ceiling finishes of the facility.    The first sentence applies to most facilities.    The second sentence applies to housing.    Use the last sentence if the "Room Requirements" or "Room Finish Schedule" are very detailed as to the ceiling finish requirements.
**********************************************************************************************************

[Primary ceiling finish shall be [24 inch by 24] [24 inch x 48] inch by 5/8 inch minimum thickness suspended acoustical panel ceiling system, except provide a suspended gypsum board ceiling in entrance lobby, restrooms [and showers].    [Acoustical panels shall have a tegular edge.    [Panels shall have a factory-applied standard washable painted finish or Type IV with factory-applied plastic membrane-faced vinyl, Form:[1] [2] [3].]

[Ceiling finish material shall be gypsum board.]

[Exposed structural systems shall be painted according to PTS Section C3040 INTERIOR COATINGS AND SPECIAL FINISHES.]

[Ceiling finishes shall be as indicated in the “Room Requirements” [Room Finish Schedule] portion of this RFP.

C3040 INTERIOR COATINGS AND SPECIAL FINISHES

********************************************************************************************************** 
NOTE:    Consider one of the following sentences, or create your own, to describe the interior coatings for the facility,    
**********************************************************************************************************

Paint all interior exposed surfaces including metal items, such as interior grilles, registers, diffusers, access panels, and panel boxes.

[Provide special [high performance architectural] [impact resistant] coatings in    [____].

[All finish coatings shall be as indicated in the "Room Requirements" [Room Finish Schedule] portion of this RFP.]

--End of Section--