*************************************************************************** 
NAVFAC                                             PTS-C20 (April 2008)
                                             ------------------------------
Preparing Activity: NAVFAC                 SUPERSEDING PTS-C20 (Aug. 2006)

PERFORMANCE TECHNICAL SPECIFICATION

***************************************************************************

SECTION C20

STAIRS
4/08

*************************************************************************** 
NOTE:  This section is intended to be used as a guide and contains requirements that are common to many different types of facilities. In addition, there may be special requirements for a particular project that are not addressed at all.  The RFP preparer will need to incorporate additional information to address these special requirements in this PTS and corresponding Part 3 ESR.  If the RFP Preparer chooses to delete building elements that are not required for the project, do not change the remaining Uniformat paragraph designations (example A102001).  Uniformat designations are unique to the products they are assigned to.  However, the subparagraphs numerical extension (example - 1.2 or a,b,c of the Uniformat designations may change if subparagraphs are deleted).

This guide specification is formatted utilizing Uniformat II, an industry recognized standard, ASTM E 1557.  When the RFP preparer chooses to add a paragraph that does not apply to an existing building element already included in the specification, refer to the Uniformat/WBS located on the NAVFAC Design-Build Website for a listing of Uniformat II designations and definitions.

NOTE:  The RFP preparer may view or hide the criteria notes in this PTS section by modifying the WORD preferences in the document pull down menu for "Hidden text". To view the criteria notes choose "Tools, Options, and View," then check the "Hidden text" box, to print the criteria notes choose "Tools, Options, and Print," then check the "Hidden text"  box.
***************************************************************************

*************************************************************************** 
NOTE:  The following Table of Contents is for navigation purposes within the document and is not intended to be part of the final edited documents

TABLE OF CONTENTS

C20     GENERAL
C20  1.1  DESIGN GUIDANCE
C20  1.2  PERFORMANCE VERIFICATION AND ACCEPTANCE TESTING
C20  1.3  DESIGN SUBMITTALS
C20  1.4  CONSTRUCTION SUBMITTALS
C2010  STAIR CONSTRUCTION
C201001  INTERIOR AND EXTERIOR STAIRS
C201001  1.1  STEEL STAIRS
C201001  1.2  ALUMINUM ALLOY PRODUCTS
C201001  1.3  FINISHES
C201001  1.4  CONCRETE STAIRS / STEPS
C201001  1.5  WOOD STAIRS
C201002  FIRE ESCAPE STAIRS
C201090  STAIR HANDRAILS, GUARDRAILS, AND ACCESSORIES
C201090  1.1  HANDRAILS
C201090  1.2  METAL LADDERS
***************************************************************************

C20 GENERAL

C20 1.1 DESIGN GUIDANCE

Provide the design and installation in accordance with the following references.  This Performance Technical Specification (PTS) adds clarification to the fundamental requirements contained in the following Government Standards.  The general requirements of this PTS section are located in PTS Section Z10, General Performance Technical Specification.

Industry standards, codes, and government standards that are referenced in the section text that are not found in the Unified Master Reference List (UMRL) in the Construction Criteria Base (CCB)  at the Whole Building Design Guide Website , are listed below for basic designation identification. Comply with the required and advisory portions of the current edition of the standard at the time of contract award.

*************************************************************************** 
NOTE:    Edit the following list of industry and government standards to coordinate with the section text additions and deletions.  List all industry and government standards in the following two paragraphs that are indicated in the section text that are not listed in the UMRL.
***************************************************************************

C20 1.1.1 Industry Standards and Codes

AISC American Institute of Steel Construction

C20 1.1.2 Government Standards

UNIFIED FACILITIES CRITERIA (UFC)

UFC 03-100-10N, Architecture

UNIFIED FACILITIES GUIDE SPECIFICATIONS (UFGS)

C20 1.2 PERFORMANCE VERIFICATION AND ACCEPTANCE TESTING

Verification of satisfactory stair performance shall be via Performance Verification Testing, as detailed in this section of the RFP.

C20 1.2.1 Field Testing for Concrete

Field Quality Control Test Reports to be submitted to DOR shall comply with ACI 301.  If concrete is found to be below the strength required in the tests, Contractor shall remove and replace that concrete and all associated building components at his own expense.

C20 1.3 DESIGN SUBMITTALS

Design Submittals shall be in accordance with Z10, General Performance Technical Specifications, UFGS section 01 33 10.05 20, Design Submittal Procedures, UFC 1-300-09N, Design Proceduresand UFC 3-100-10N, Architecture.

*************************************************************************** 
NOTE:  NAVFAC has made every effort to use commercial standards in the PTS sections.  This PTS section is designed to only use commercial standards.  If project requirements dictate the use of a UFGS sections as a standard, add a paragraph here listing the required UFGS section. State in the paragraph that the DOR must edit this UFGS section in accordance with PTS Z10 and submit it as a part of the design submittal.
***************************************************************************

C20 1.4 CONSTRUCTION SUBMITTALS

Submit construction submittals in accordance with PTS Section Z10, General Performance Technical Specification.  In addition to the Z10 requirements the Designer of Record (DOR) shall approve the following construction submittals as a minimum:

Stairs, handrails.

C2010 STAIR CONSTRUCTION

C201001 INTERIOR AND EXTERIOR STAIRS

Provide primed and painted steel stairs with concrete filled pans or cast-in-place concrete stairs for industrial and commercial construction. Design load shall not be less than 100 PSF (4.8 kPa) for live load, and 300 pounds (136 kg) for concentrated loads.  Required means of egress stairs shall conform to NFPA 101. Provide steel guard and handrails.

C201001 1.1 STEEL STAIRS

Design shall conform to AISC S335 or AISC S342L.

C201001 1.1.1 Materials

a.	Structural Carbon Steel - ASTM A 36/A 36M

b.	Structural Tubing - ASTM A 500

c.	Steel Pipe - ASTM A 47

d.	Gratings - Gray cast iron ASTM A 48, Class 40

e.	Metal plank grating, non-slip requirement, FS RR-G-1602 aluminum ASTM B 209, 6061-T6; steel ASTM A 653/ A 653M, G90.

f.	Floor Plates, Patterned - ASTM A 786/A 786M, 14 gage.

g.	Anchor Bolts - ASTM A 307

h.	Galvanized Structural Steel - All steel exposed to the environment or direct water contact shall be galvanized in accordance with ASTM A 123 /A123M, ASTM A153/A153M, and ASTM A653/A653M, Z275 (G90) coating.  Galvanize all components after fabrication in accordance with ASTM A385. Fabricate all steel components in the largest units practical using bolted connections for field assembly.  Repair damage to, or voids in, galvanizing in accordance with ASTM A780, Annex A1 or Annex A3.

C201001 1.2 ALUMINUM ALLOY PRODUCTS

Conform to ASTM B 209 for sheet plate, ASTM B 221 for extrusions and ASTM B 26/B 26M or ASTM B 108 for castings.  Aluminum extrusions shall be at least 1/8-inch (3.2 mm) thick and aluminum plate or sheet at least 0.050 inch (1.27 mm) thick.

C201001 1.3 FINISHES

C201001 1.3.1 Galvanizing

Hot-dip galvanizing: ASTM A 123/A123M, ASTM A 153/A 153M or ASTM A 653/A 653M, G90, as applicable.

C201001 1.3.2 Aluminum Finishes

Protect by plating, Class I anodic coatings, or 70% polyvinylidene fluoride organic coatings.  See PTS Section C30 for additional coatings/finish information.

C201001 1.3.3 Safety Treads

NAAMM BG steel, Type W (welded).

C201001 1.3.4 Other Coatings

See PTS Section C30, Interior Finishes, for painted finishes.

C201001 1.4 CONCRETE STAIRS / STEPS

Provide interior or exterior concrete steps and stair with non-slip finish. For interior stairs, provide rubber or other finish treads.  For exterior stairs, provide cast-in-place abrasive nosing. Provide steel guard and handrails as necessary. Fire stairs shall comply to NFPA 101.

C201001 1.4.1 Materials

a.	Concrete - ACI 211.1, ACI 301/301M, and ACI 318/318M, with a compressive strength of 3,000 psi (20,680 kPa) or greater. Concrete Mix Design shall be suitable for the job conditions.

b.	Reinforcements - Bars, fabrics, connectors, and chairs shall be galvanized.

c.	Reinforcing Bars - ACI 301/301M

d.	Welded Wire Fabric - ASTM A 185 or ASTM A 497

e.	Cast Aluminum Safety Nosings – For exterior concrete stairs, provide safety nosings of cast aluminum with abrasive surfaces or with abrasive inserts.

C201001 1.5 WOOD STAIRS

Wood stairs may be used only for residential construction.  Provide wood treads of minimum 1-1/4 inches thickness, of clear red or white oak.  Risers shall be nominal one-inch finish lumber.  Provide natural finish equivalent to one coat of sealer and two coats of varnish on all exposed surfaces.

C201002 FIRE ESCAPE STAIRS

*************************************************************************** 
NOTE: Do not use fire escape stairs in new construction.  Fire escape stairs shall not constitute any of the required means of egress, with exception granted to some existing buildings by the NFPA.  New fire escape stairs shall be only be permitted to be erected on existing buildings where the authority having jurisdiction has determined that outside stairs are impractical.
***************************************************************************

Design fire escapes of the type and arrangement to conform to Fire Escape Stairs, of NFPA 101, Life Safety Code.  Escape stairs shall be of steel or aluminum, conforming to the requirements of this specification section.

C201090 STAIR HANDRAILS, GUARDRAILS, AND ACCESSORIES

C201090 1.1 HANDRAILS

Design handrails in accordance with the IBC, except delete the handrail design load reduction code exceptions for residential, prisons, industrial, high hazard, and storage facilities.  NAAMM PR, provide the same size rail and post.  Provide series 300 stainless steel pipe collars. Factory coat all metal railings (except for ornamental metals such as brass, bronze, stainless steel, and nickel-silver) with a high performance coating in accordance with AAMA 2605, with a minimum coating thickness of 1.2 mils unless otherwise noted.

C201090 1.1.1 Steel Handrails

Provide steel handrails, including inserts in concrete, steel pipe conforming to ASTM A 53 or structural tubing conforming to ASTM A 500, Grade A or B of equivalent strength.  Railings shall be hot-dip galvanized and shop painted for exterior applications and primed and shop painted for interior applications.  Railing may be unpainted hot-dip galvanized in industrial areas.

C201090 1.1.2 Aluminum Handrails

Provide aluminum pipe railing conforming to ASTM B 429 or square aluminum semi-hollow tube conforming to ASTM B 221.  Railings shall be coated with a high performance coating or anodized in accordance with AAMA 611, Class I.

C201090 1.1.3 Ornamental Handrails

Provide ornamental railings. Provide anchorage and fasteners as recommended by the product manufacturer. Railing system shall conform to ASTM E 985, minimum concentrated test load requirement.

C201090 1.1.4 Glass Handrails

Provide glass railings consisting of continuous 1/2-inch (13 mm) beveled tempered glass structural balusters with continuous railing cap and bottom shoe molding. Railing cap and shoe molding shall be 6063-T52 aluminum, type 304 stainless steel, brass, or bronze.

C201090 1.1.5 Wood Handrails

Wood handrails shall only be used for residential construction.  Provide wood handrails of pre-finished natural hardwood. Wood shall be coated with hard acrylic finish to withstand indentations.

C201090 1.2 METAL LADDERS

C201090 1.2.1 Metal Ladders

Provide vertical ladders conforming to Section 7 of 29 CFR 1910.27.

C201090 1.2.2 Installation

Offset distance from the rungs to the finished wall surface not less than 7 inches (175 mm). Provide heavy clip angles riveted or bolted to the stringer and drilled for not less than two 1/2-inch (12 mm) diameter expansion bolts as indicated. Provide intermediate clip angles not over 48 inches (1200 mm) on center.

C201090 1.2.3 Ladder Cages

Where the height of the ladder is greater than 20 feet (6000 mm), provide a cage to conform to 29 CFR 1910.27.

a.	Cage fabrication – Provide attachments for fastening bands to the side rails of ladders or directly to the structure.

-- End of Section --