*************************************************************************** 
NAVFAC                                                PTS-H60 (April 2008)
                                             -----------------------------
Preparing Activity: NAVFAC               SUPERSEDING PTS-H60 (March 2007)

PERFORMANCE TECHNICAL SPECIFICATION

***************************************************************************

SECTION H60

WATERFRONT DEMOLITION
4/08

*************************************************************************** 
NOTE:  This section is intended to be used as a guide and contains requirements that are common to many different types of facilities; however, not all requirements and equipment items will be applicable to all projects.  In addition, there may be special requirements for a particular project that are not addressed at all.  The RFP preparer may have to incorporate additional information to address these special requirements in this PTS and corresponding Part 3 ESR.  If the RFP preparer chooses to delete elements that are not required for the project, do not change the remaining Uniformat paragraph designations (example - A102001). Uniformat designations are unique to the products they are assigned to. However, the subparagraph numerical extensions (example - 1.2 or a,b,c) of the Uniformat designations may change if subparagraphs are deleted.

This guide specification is formatted utilizing Uniformat II, an industry recognized standard, ASTM E 1557.  When the RFP preparer chooses to add a paragraph that does not apply to an existing building element already included in the specification, refer to the Uniformat/WBS located on the NAVFAC Design-Build Website for a listing of Uniformat II designations and definitions.

NOTE:  The RFP preparer may view or hide the criteria notes in this PTS section by modifying the WORD preferences in the document pull down menu for "Hidden text". To view the criteria notes choose "Tools, Options, and View," then check the "Hidden text" box, to print the criteria notes choose "Tools, Options, and Print," then check the "Hidden text" box.
***************************************************************************

*************************************************************************** 
NOTE:  The Table of Contents is intended for navigation purposes only for the RFP writer and should not show up in the printed document.

TABLE OF CONTENTS

H60  GENERAL
H60  1.1  NARRATIVE
H60  1.2  WATERFRONT DEMOLITION DESIGN GUIDANCE
H60  1.3  QUALITY ASSURANCE
H60  1.4  DESIGN SUBMITTALS
H60  1.5  CONSTRUCTION SUBMITTALS
H6010  IN-WATER OR OVER-WATER DEMOLITION
H6020  NON-IN-WATER OR NON-OVER-WATER WATERFRONT DEMOLITION
H6030  HAZARDOUS COMPONENTS ABATEMENT
***************************************************************************

H60 GENERAL

H60 1.1 NARRATIVE

This Section must be used in conjunction with all parts of the Design Build (D/B) Request for Proposal (RFP) to determine the full requirements of this solicitation.

This Section includes the requirements for the design and construction of waterfront demolition, described in ESR H60, Waterfront Demolition, of Part 3 and as identified on the RFP drawings provided in Part 6.

H60 1.2 WATERFRONT DEMOLITION DESIGN GUIDANCE

*************************************************************************** 
NOTE:  This Performance Technical Specification (PTS) has been developed to utilize certain UFGS sections for prescriptive requirements.  However, there may be rare occasions when additional prescriptive specifications may be identified by the Engineering Systems Requirements (ESR) to be edited by the Contractor's Designer of Record. If other UFGS sections or standards are to be referenced in this PTS section, list those not covered by the UMRL in the following two paragraphs.

If the product or system is new and not covered in the PTS, provide a new paragraph and heading.  If the product or system is presently covered in the PTS but is being changed by the reference to the UFGS, edit the PTS paragraphs to eliminate redundancy or conflicts.  In either case, identify the UFGS section in the products and materials (UNIFORMAT II/WBS level 4) text of the PTS as follows:

 "Uniformat II/WBS Number – Paragraph Number Paragraph Heading
The Designer of Record shall utilize UFGS Section Number, "Section Title" for the project specification, and shall submit the edited specification section as a part of the design submittal for the project."
***************************************************************************

Provide the design and installation in accordance with the following references.  This Performance Technical Specification (PTS) adds clarification to the fundamental requirements contained in the following Government Standards.  The general requirements of this PTS Section are located in PTS Section Z10, General Performance Technical Specification.

H60 1.2.1 Government Standards

UNIFIED FACILITIES CRITERIA (UFC)

UFC 3-800-10N, Environmental Engineering for Facility Construction

UNIFIED FACILITIES GUIDE SPECIFICATIONS (UFGS)

UFGS Section 02 83 13.00 20, Lead In Construction

*************************************************************************** 
NOTE:  Add project-specific design information below.  Do not include information that is readily available to the contractor from UFC or other sources.
***************************************************************************

H60 1.2.2 General Requirements

H60 1.2.2.1 General Demolition

The work includes demolition, salvage of identified items and materials and removal of resulting rubbish and debris.  Rubbish and debris shall be removed from Government property daily, unless otherwise directed.  Materials that cannot be removed daily shall be stored in areas specified in the Demolition Plan.  Remove indicated existing elements of waterfront facilities as identified on drawings provided in Part 6 and as required to be determined by the Contractor.

H60 1.2.2.2 Utilities

Remove existing utilities as identified in ESR H60, Waterfront Demolition, and on RFP drawings provided in Part 6 and terminate in a manner conforming to the nationally recognized code covering the specific utility.

H60 1.2.2.3 Dust Control

Prevent the spread of dust and debris and avoid the creation of a nuisance or hazard in the surrounding area.

H60 1.2.2.4 Traffic Control

Where pedestrian and vehicle safety is endangered, use traffic barricades.

H60 1.2.2.5 Burning

Burning will not be permitted.

H60 1.2.2.6 Lead Based Paint

Refer to the "Lead Based Paint" paragraph in PTS Section F20, Selective Building Demolition.

H60 1.2.2.7 Mercury and Low Level Radioactive (LLR) Component

Refer to the "Mercury and LLR Components" paragraph in PTS Section F20.

H60 1.2.2.8 PCB's

Refer to the "PCB's" paragraph in PTS Section F20, Selective Building Demolition.

H60 1.2.2.9 Ozone Depleting Substance (ODS)

Refer to the "ODS" paragraph in PTS Section F20, Selective Building Demolition.

H60 1.2.2.10 Salvage Material

All demolished material will become the property of the Contractor except for the salvage items identified in ESR H60, Waterfront Demolition, in Part 3.  Salvage items will become the property of the Government.

H60 1.2.2.11 Permit and Environmental Control

Waterfront demolition work shall meet the permit and environmental control requirements of Section 01 57 19.00 20, Temporary Environmental Controls for Design-Build, in Part 2.  In-water and over-water demolition shall not commence until the required USACOE permits are obtained and shall meet the conditions stipulated in the USACOE permits.

H60 1.3 QUALITY ASSURANCE

Qualifications, certifications, and Test Plans indicated herein shall be submitted 45 calendar days prior to the expected date of execution. Notify the Contracting Officer 14 calendar days prior to all testing. Submit test results within 7 calendar days of completion of testing.

The Designer of Record is responsible for approving the submittals listed below.

H60 1.4 DESIGN SUBMITTALS

Design submittals shall be in accordance with PTS Section Z10, General Performance Technical Specification, 01 33 10.05 20, Design Submittal Procedures, UFC 1-300-09N, Design Procedures, UFC 3-800-10N, Environmental Engineering for Facility Construction.

H60 1.5 CONSTRUCTION SUBMITTALS

The general requirements of this PTS Section are located in PTS Section Z10, General Performance Technical Specification.  Construction submittals shall be in accordance with 01 33 00.05 20, Construction Submittal Procedures.

H6010 IN-WATER OR OVER-WATER DEMOLITION

Demolition of in-water or over-water substructure, superstructure, deck and coastal protection components identified on the RFP drawings provided in Part 6 or as required based on the Contractor's design shall be performed meeting the permit and environmental control requirements specified above.

H6020 NON-IN-WATER OR NON-OVER-WATER WATERFRONT DEMOLITION

For non-in-water or non-over-water waterfront demolition, refer to PTS Section F20, Selective Building Demolition.

H6030 HAZARDOUS COMPONENTS ABATEMENT

Refer to the "Hazardous Components Abatement" paragraph in Section F20, Selective Building Demolition.

---End of Section---