6. ENGINEERING SYSTEMS REQUIREMENTS

C10 INTERIOR CONSTRUCTION

********************************************************************************************************** 
SYSTEMS REQUIREMENTS
INTERIOR CONSTRUCTION TEMPLATE 12/08
 
Instructions for using this template: There are template files for each UNIFORMAT Level 2 Group Elements. This template is for Group Element C10-INTERIOR CONSTRUCTION. Text such as this is hidden text that will not print when the hidden text box in "Print/Options" is un-checked.
 
The Architectural Team Member should edit this template for the requirements of the project. The SYSTEMS REQUIREMENTS are intended to define items that are required throughout the facility or on a system wide basis that is common to several rooms. Room-specific requirements are defined in the ROOM REQUIREMENTS section. Coordinate with the lead programmer for ROOM REQUIREMENTS. Editing is required where brackets [ ] appear. Delete all building elements that are not required for the project. If additional elements or sub-elements are required for the project that do not appear in the template, refer to the NIST UNIFORMAT II publication for additional building element numbers and descriptions. The Uniformat II Work Breakdown Structure can be found at www.wbdg.org/ndbm/ . Coordinate with the PERFORMANCE TECHNICAL SPECIFICATION SECTION C10 to ensure that performance requirements are provided for all of the Building Elements listed here and that paragraph numbering matches.
**********************************************************************************************************

SYSTEM DESCRIPTION
********************************************************************************************************** 
NOTE:    Edit the following paragraph based on the project specific information received from the User.    Fill in the blank with the function of the building, such as residence, barracks, administrative office building, training facility, operations building, fire station, etc.
**********************************************************************************************************

Interior construction includes interior partitions, interior doors, and fittings.

********************************************************************************************************** 
NOTE:    It is preferred to provide this information under Chapter 2, "Project Objectives" or Chapter 5, "Room Requirements."
**********************************************************************************************************

Provide durable construction appropriate to the buildings use as an __________________________.    Acoustic properties of materials, as well as durability, shall be considered during material selection.

GENERAL SYSTEMS REQUIREMENTS 
 [[Areas of] the Project [is] [are] subject to abuse and require that "impact Resistant" systems be provided.] See "Room Requirements" for specific requirements on "Partitions", "Interior Doors", and "Fittings".

C1010 PARTITIONS

********************************************************************************************************** 
NOTE:    Consider one of the following sentences, or create your own, to describe the interior partitions of the facility.    The first two sentences would be applicable to a barracks, or industrial facility, where extreme durability is required.    The third sentence would apply to an administrative office building.    The fourth sentence would apply to housing or a small facility under 5000 square feet.
**********************************************************************************************************

[All interior partitions shall be concrete masonry.]

[All interior partitions shall be cast-in-place concrete.]

[All interior partitions shall be constructed of metal studs with gypsum board on each side.]

[All interior partitions shall be constructed of wood studs with gypsum board each side.]

[All interior partitions shall be either concrete masonry or metal studs with gypsum board each side.]

[Concrete mixture shall provide an average compressive strength of [3000 PSI (20,680 kPa)] [_____] and meet or exceed ACI 301/301M.]

C101001 FIXED PARTITIONS

Provide fixed interior partitions, except where demountable or retractable partitions are specifically required by the "Room Requirements." Sound-rated partition assemblies shall have a minimum Sound Transmission Coefficient (STC) of [36] [42] [__] in accordance with ASTM E 90 or ASTM E 413 for frequency data.

[Glass masonry units shall transmit [75] [___] percent light.]

C101002 DEMOUNTABLE PARTITIONS

[Demountable partitions include [sound rated,] full height (floor to ceiling) solid partitions [and wire mesh partitions].    Sound-rated partition assemblies shall have a minimum Sound Transmission Coefficient (STC) of [36] [42] [__] in accordance with ASTM E 90 or ASTM E 413 for frequency data.    Extend wire mesh partitions meant for security purposes to the structural ceiling, or provide a wire mesh ceiling. [Demountable partitions may also be funded as part of FF&E.]

C101003 RETRACTABLE PARTITIONS

[Provide retractable partitions to include [operable panel partitions] [and] [accordion folding partitions].    Sound-rated partition assemblies shall have a minimum Sound Transmission Coefficient (STC) of [36] [42] [__] in accordance with ASTM E 90 or ASTM E 413 for frequency data.]

C101004 INTERIOR GUARDRAILS AND SCREENS

Provide balustrades where required by code.    Provide screens where required to prohibit view of a particular area.

C101005 INTERIOR WINDOWS

Provide interior windows of [aluminum] [wood] [plastic] [hollow metal] [bullet resistant metal], [fixed][or][operable].

Bullet-resistant windows shall meet U.L. Classification [Rating Level 1 through 8], as required for the installation. Each window shall be a complete factory-assembled unit with glass factory or field installed.

C101006 GLAZED PARTITIONS & STOREFRONTS

[Provide glazed storefront system.]

C101007 INTERIOR GLAZING

Interior glazing shall be clear glass, [wire glass,] [patterned glass,] [laminated glass,] [bullet resistant glass,] [tempered glass,] [plastic glazing.]

C1020 INTERIOR DOORS

********************************************************************************************************** 
NOTE:    Consider one of the following sentences, or create your own, to describe the interior doors of the facility.    The first sentence applies to industrial facilities, where extreme durability is required.    The second sentence applies to most other facilities.    The third sentence applies to housing.    Use the fourth sentence if the "Room Requirements" are very detailed as to the door requirements.
**********************************************************************************************************

C102001 STANDARD INTERIOR DOORS

[All interior doors shall be [flush][stile and rail paneled][to match the appearance of the existing doors _______.]

[All interior doors and frames shall be hollow metal.]

[All interior doors shall be wood[, except where hollow metal doors are required to meet fire rating].    All interior door frames shall be hollow metal.]

[All interior doors shall be wood or hollow metal in hollow metal frames.]

********************************************************************************************************** 
NOTE:    Wood door frames are only acceptable in residential construction.
**********************************************************************************************************

[All interior doors and frames shall be wood.]

[Stile and rail wood doors shall be WDMA I.S.6A-01, [premium] custom grade, [extra] heavy duty.]

[Flush wood doors shall be WDMA I.S.1A-04, [premium] custom grade, [extra] heavy duty.] [Flush wood doors shall be WDMA I.S.-97 [PC-5 5-[ly particleboard core][SCLC-5 5-ply structural composite lumber core].

[Building standard doors to comply with WDMA I.S.1A Premium Grade. Doors adjacent to paneling or millwork to comply with corresponding AWI millwork grade.]

********************************************************************************************************** 
NOTE:    Wood veneers shall be hard- or softwood veneers cut for the best presentation for natural finishing of doors.    Only use random match with opaque painted doors.
**********************************************************************************************************

[Veneer for flush doors shall be [species] in [veneer match].]

********************************************************************************************************** 
NOTE:    Wood Door Factory Finish: High quality - Conversion varnish alkyd urea; highest quality - catalyzed polyurethane, or acrylated uv curable epoxy.    Field finish shall be as shown in PTS C30.    
**********************************************************************************************************

Doors shall have [Factory Finish of AWI Quality Standards Section 1500, specification for [Conversion varnish alkyd urea] [catalyzed polyurethane] [or] [acrylated uv curable epoxy] [field finish].

C102002 GLAZED INTERIOR DOORS

Provide vision glazing in doors where it is required by the “Room Requirements” portion of this RFP, or it is deemed advantageous to be able to see through the door, either for safety of pedestrian traffic, or other functional reason.

C102003 FIRE DOORS

[Provide interior fire doors.]

C102004 SLIDING AND FOLDING DOORS

********************************************************************************************************** 
NOTE:    Confirm the acceptable type of closet door with the activity.
**********************************************************************************************************

[Provide paneled louvered doors of [premium] [custom] grade, conforming to WDMA I.S.6A-01, [premium] [custom] grade, heavy duty.]

[Closet doors shall be [sliding] [bi-folding]].

C102005 INTERIOR OVERHEAD DOORS

[Provide interior overhead doors.]

C102006 INTERIOR GATES

[Provide interior gates.]

C102007 INTERIOR DOOR HARDWARE

[Provide special door hardware, such as [combination locks] [card key system] at [_________].

[Provide card key type access units for specialized entries.    Provide lithium battery powered, magnetic stripe keycard locksets that are [ANSI/BHMA A156.13, Series 1000, Grade 1, mortise] [ANSI/BHMA A156.2, Series 4000, Grade 1, cylindrical locks, tamper resistant, UL listed with 1 inch (25 mm) throw deadbolt, 3/4-inch (19 mm ) throw latch bolt, auxiliary dead-locking latch, and 2-3/4 inch (68.75 mm) backset.]]

[Existing locks were manufactured by [_____] and [do not] have interchangeable cores.] [Lock cores shall match the BEST system.]

Door hardware finish shall be [chrome-plated brass or bronze, or stainless steel] [brass or bronze].

C102090 OTHER INTERIOR SPECIALTY DOORS

[Provide [___] interior specialty doors.]

C102091 OTHER INTERIOR PERSONNEL DOORS

[Provide [___] interior personnel doors.]

C1030 SPECIALTIES

C103001 COMPARTMENTS, CUBICLES, & TOILET PARTITIONS

Provide [stainless steel] [plastic laminate] [solid plastic] [phenolic core] [or metal with enamel finish] toilet partitions in all toilet rooms with more than one water closet or urinal.    Provide toilet accessories as indicated in Chapter 3, “Room Requirements” portion of this RFP.

C103002 TOILET AND BATH ACCESSORIES

[Provide toilet and bath accessories.]

C103003 MARKER BOARDS AND TACK BOARDS

[Provide marker boards and tack boards.]

C103004 IDENTIFYING DEVICES

All necessary interior signage shall be incorporated as part of the architectural drawings.    Interior signage is not collateral equipment. Interior signage shall demonstrate complete coordination with the facility design, SID and FF&E submittals.    Provide interior directional signage as required for facility wayfinding.    Provide an identifying device at each interior door.    Signs must meet ADA requirements.

C103005 LOCKERS

Provide lockers [with special bases of [____].    Lockers shall be [wire] [metal construction and enamel finish] [high density polyethylene/solid plastic] [or plastic laminate].

C103006 SHELVING

[Provide [steel utility] [plastic laminate clad] shelving.]

C103007 FIRE EXTINGUISHER CABINETS

[Provide fire extinguisher cabinets.    Cabinet shall be semi-recessed in new construction and surface-mounted in new mechanical/electrical spaces and existing wall construction.      Cabinets shall be coordinated with interior finishes]

C103008 COUNTERS

Provide [solid acrylic] [plastic laminate] [wood] [stone] [stainless steel] [or tile] counter tops and back splashes.

C103009 CABINETS

Provide cabinetry and millwork items with associated accessories.    Cabinetry shall be AWI [premium] [custom] grade and have concealed hinges with adjustable standards for shelves.    All exposed surfaces will be [covered with high pressure plastic laminate clad] or [hardwood veneer with exposed edges of solid hardwood].

Provide specific cabinetry as shown on the Room Requirements Sheets.

C103010 CASEWORK

[Provide all built-in premanufactured metal cabinetry for specialized functions such as laboratories, libraries, medical and dental facilities.][Casework shall comply with Mil Std 1691.]

Provide specific casework as shown on the Room Requirements Sheets.

C103011 CLOSETS

Provide [premanufactured millwork closets] [prefabricated coat closets] (for schools and dormitories).

C103012 FIRESTOPPING PENETRATIONS

[Provide all sleeves, caulking, and flashing for firestopping penetrations.]

C103013 SPRAYED FIRE-RESISTIVE MATERIALS

[Provide sprayed fire-resistive materials to the building’s framework to prevent structural failure]

C103014 ENTRANCE FLOOR GRILLES AND MATS

Provide [recessed pan] [surface] floor mats at [main] [all] building entrances.

C103015 ORNAMENTAL METAL WORK

[Provide ornamental [metalwork][stair handrails.]

C103090 OTHER INTERIOR SPECIALTIES

Provide [wall or ceiling mounted] [above ceiling mounted] motorized projection screen as approved by client.

Provide [above ceiling mounted projector] [ceiling mounted projector] as approved by client.

[Provide pull down or manual projection screens shall be provided in lieu    of motorized projection screens as approved by client.]

--End of Section--